NOTICE TO MARINERS

PETERHEAD PORT AUTHORITY

CHARTED DEPTHS AND BERTH INFORMATION

NOTICE IS HERBY GIVEN, that the latest charted depths and quayside information can be found in the attached table below

Marine Department

8th December 2025

Berth Details - Bathymetric Survey August 2025									 (MHWS 3.8m above chart datum)

| Berth Name & Nos | Length | Bearing | Width | Height Quay (Above Chart Datum) | Least Depth (Below Chart Datum) | Fresh Water | Fuel | Explosives (Kg) | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Princess Royal Jetty | 173m | 223° - 043° |  |  |  |  |  |  |  |
| 1. | 95 |  | 23m | 6.2m | 6.3m (Northern 90m) | Yes | Yes | 650 | Max Vessel 90m |
| 2. | 170 |  | 23m | 6.2m | 6.6m (Northern 130m) | Yes | Yes | 1200 | 25m from South end of the berth is 6.1m. Approach 6.3m |
| ASCO South Base | 486m | 313° - 133° |  |  |  |  |  |  |  |
| 4. | 98 |  | 18m | 6.2m | 6.4m | Yes | Yes | 50 |  |
| 5. | 96 |  | 18m | 6.2m | 6.4m | Yes | Yes | 50 |  |
| 6. | 97 |  | 18m | 6.2m | 6.2m | Yes | Yes | 50 |  |
| 7. | 94 |  | 18m | 6.2m | 6.2m | Yes | Yes | 300 | No 8 Shelves to 4.0m beyond Knuckle |
| 8. | 101 |  | 18m | 6.2m | 5.9m | Yes | Yes | 450 |  |
| South Breakwater | 397 | 215° - 035° |  |  |  |  |  |  | Depths on routes not inc |
| 10. | 101 |  | 17m | 7.2m | 6.4m | No | No | 10,000 |  |
| 11. | 101 |  | 17m | 7.2m | 6.9m | No | No | 13,000 |  |
| 12. | 101 |  | 17m | 7.2m | 7.1m | No | No | 8000 |  |
| North Breakwater | 255 | 199° - 019° |  |  |  |  |  |  | 6 x (3.3m x 2.0m) Yoko Fenders 8.9m on Approach |
| 13. | 120 |  | 14m | 7.2m | 9.5m | No | No | 4000 |  |
| 15. | 120 |  | 14m | 7.2m | 11.0m | Yes | Yes | 2000 | Approaches to Berth 9.8m |
| North Base Jetty | 100m | 346° - 166° |  |  |  |  |  |  |  |
| 18. | 122m |  | 15m | 7.74m | 5.8m (Southern 80m) | Yes | Yes | Nil | ** 4.4m at 85m |
| Tanker Jetty (Ldg Hd) | 40m | 215° - 035° |  |  |  |  |  |  |  |
| Max Size V/L 250m | DWT | 50,000 t | 15m | 7.2m | 10.8m | Yes | No | 250 | 3 x (6.5m x 3.3m) Yoko |
| Smith’s Quay | 120m | 089° - 269° | 40m | 6.2m | 9.4m | Yes | Yes | 50 | Max LOA 160 metres |
| West Quay Jetty East & West | E 60m W 85m |  | 16m | 6.2m 6.2m | 9m | Yes | Yes | Nil |  |
| Merchants Quay | 140m |  | 65m | 6.2m | 7.1m | Yes | Yes | Nil | Max vessel 120m |
| Albert Quay | 255m | 250° - 070° |  | 6.2m | 8.7m (westerly 180m) 7.8m (Easterly 75m) | Yes Yes | Yes Yes | 50 Nil |  |
| East Quay | 88m |  | 15m | 6.2m | 7.0m – 8.0m | Yes | Yes | Nil |  |